Yaesu FT-857D Cheat Sheet

| No | Menu Item | Function | Available Values | Default |
| --- | --- | --- | --- | --- |
| 001 | EXT MENU | Enables/Disables the extended Menu Mode. | ON/OFF | OFF |
| 002 | 144 MHz ARS | Activates/deactivates the Automatic Repeater Shift feature when operating on the 144 MHz band. | ON/OFF | *1 |
| 003 | 430MHz ARS | Activates/deactivates the Automatic Repeater Shift feature when operating on the 430 MHz band. | ON/OFF | *1 |
| 004 | AM&FM DIAL | Enabling/disabling the DIAL knob on the AM and FM modes. | ENABLE/DISABLE | DISABLE |
| 005 | AM MIC GAIN | Adjusts the microphone gain level for the AM mode. | 0-100 | 50 |
| 006 | AM STEP | Selects the tuning steps for the MEM/VFO CH knob on the AM mode. | 2.5/5/9/10/12.5/25kHz | *1 |
| 007 | APO TIME | Selects the Auto Power Off time (time before power goes off). | OFF/1h-6h | OFF |
| 008 | ARTS BEEP | Selects the ARTS beep mode. | OFF/RANGE/ALL | RANGE |
| 009 | ARTS ID | Enables/disables the CW identifier during ARTS operation | ON/OFF | OFF |
| 010 | ARTS lDW | Stores your call sign into the CW identifier. | - | YAESU |
| 011 | BEACON TEXT 1 | Stores the message for the Beacon. | - | - |
| 012 | BEACON TIME | Selects the interval time (between message and message). | OFF/ 1 sec-255 sec | OFF |
| 013 | BEEP TONE | Selects the beep frequency | 440/880/1760Hz | 880Hz |
| 014 | BEEP VOL | Selects the beep volume level. | 0-100 | 50 |
| 015 | CAR LSB R | Sets the Rx Carrier Point for LSB | -300 ~+300Hz | 0 Hz |
| 016 | CAR LSB T | Sets the Tx Carrier Point for LSB | -300 ~+300Hz | 0 Hz |
| 017 | CAR USB R | Sets the Rx Carrier Point for USB | -300 ~+300Hz | 0 Hz |
| 018 | CAR USB T | Sets the Tx Carrier Point for USB | -300 ~+300Hz | 0 Hz |
| 019 | CAT RATE | Sets the transceiver's circuitry for the CAT baud rate to be used. | 4800 bps/9600 bps/38400 bps | 4800bps |
| 020 | CAT/LIN/TUN | Selects the device which is connected to the CAT/ LINEAR jack on the rear panel. | CAT/LINEAR/TUNER | CAT |
| 021 | CLAR DIAL SEL | Defines the "control" knob to be used for setting of the clarifier offset frequency. | SEL, MAIN | SEL |
| 022 | CW AUTO MODE | Selects whether the KEY jack shall be "Enabled" or "Disabled'" while using the SSB/FM modes. | ON/OFF | OFF |
| 023 | CW BFO | Sets the CW carrier oscillator injection side on the CW mode. | USB/LSB/AUTO | USB |
| 024 | CW DELAY | Sets the receiver recovery time during pseudo VOX CW semi-break-in operation. | FULL/ 300 ~ 3000 ms | 250msec |
| 025 | CW KEY REV | Sets the keyer paddle's wiring configuration. | NORMAL/REVERSE | NORMAL |
| 026 | CW PADDLE | Enables/disables CW keying by the microphone's [UP/DWN] keys. | ELEKEY/MIC KEY | ELEKEY |
| 027 | CW PITCH | Setting of the pitch of the CHW side tone, BFO offset, and CW filter center frequencies. | 400~800Hz | 700Hz |
| 028 | CW QSK | Selects the time delay between when the PTT is keyed and the carrier is transmitted during QSK Operation when using the internal keyer. | 10/15/20/25/30ms | 10ms |
| 029 | CW SIDE TONE | Setting of the CW sidetone volume level. | 0-100 | 50 |
| 030 | CW SPEED | Sets the sending speed for the built-in Electronic keyer | 4~60wpm(1 wpm/step)/ 20~300cpm (5 cpm/step) | 12wpm (60cpm) |
| 031 | CW TRAINING | Sends random Morse Code five--character groups via the sidetone. | N,A,AN | N |
| 032 | CW WEIGHT | Sets the Dot/Dash ratio for the built-in electronic keyer | 1:2.5~1:4.5 | 1:3.0 |
| 033 | DCS CODE | Setting of the DCS code. | 104 Standard DCS codes | 023 |
| 034 | DCS INV | Selects 'Normal' or "lnverted" DCS coding. | Tn-Rn/Tn-Rn/Tn-Rn / Tn-Rn | Tn-Rn |
| 035 | DIAL STEP | Setting of the DIAL knob's tuning speed. | FINE/COARSE | FINE |
| 036 | DIG DISP | Defines the displayed frequency offset during DIG (USER-L or USER-U) mode operation | -3000~+3000 Hz | 0 Hz |
| 037 | DIG GAIN | Adjusts the audio input level from terminal equipment during DIG (Digital) mode operation. | 0-100 | 50 |
| 038 | DIG MODE | Selects the mode and sideband in the DIG (Digital) mode. | RTTY-L/RTTY-U/PSK31-L/ RTTY-L/PSK31-U/ USER-L/USER-U | RTTY-L |
| 039 | DIG SHIFT | Defines the carrier frequency offset during DIG (USER-L or USER-U) mode operation. | -3000~+3000Hz | 0 Hz |
| 040 | DIG VOX | Sets the gain of the VOX circuitry's input level for the DIG mode | 0-100 | 0 |
| 041 | DISP COLOR | Selects the illumination color for each operating status | - | FIX:26 |
| 042 | DISP CONTRAST | Setting of the display contrast level | 1~13 | 5 |
| 043 | DISP INTENSITY | Setting of the display brightness level. | 1 (Dim) ~ 3 (Bright) | 3 |
| 044 | DISP MODE | Sets up the LCD Lamp mode | OFF/AUT01/AUT02/0N | AUT02 |
| 045 | DSP BPF WIDTH | Setting of the bandwidth for the DSP CW audio filter. | 60/120/240 Hz | 240 Hz |
| 046 | DSP HPF CUTOFF | Adjusts the low-cut characteristics of the DSPHPF filter. | 100~1000 Hz | 100 Hz |
| 047 | DSP LPF CUTOFF | Adjusts the high-cut characteristics of the DSP LPF filter. | 1000~6000 Hz | 6000 Hz |
| 048 | DSP MIC EQ | Sets the DSP microphone equalization pattern | OFF/LPF/HPF/BOTH | OFF |
| 049 | DSP NR LEVEL | Setting of the degree of DSP Noise Reduction. | 1~16 | 8 |
| 050 | EMERGENCY | Enables Tx/Rx operation on the Alaska Emergency Channel, 5167.5 kHz | ON/OFF | OFF |
| 051 | FM MIC GAIN | Adjusts the microphone gain level for the FM mode. | 0-100 | 50 |
| 052 | FM STEP | Selects the tuning steps for the MEM/VFO CH knob in the FM mode | 5/6.25/10/12.5/15/20/25/50 kHz | *2 |
| 053 | HOME->VFO | Enables/disables the moving of HOME channel data to the VFO. | ON/OFF | ON |
| 054 | LOCK MODE | Selects the operation of the front panel's LOCK key | DIAL/FREQ/PANEL/ALL | DIAL |
| 055 | MEM GROUP | Enables/disables the memory-grouping feature. | ON/OFF | OFF |
| 056 | MEM TAG | Stores Alphanumeric 'Tags’ for the memory channels. | - | - |
| 057 | MEM/VFO DIAL MODE | Selects the function which is engaged when you press the MEM/VFO CH knob. | CW SIDETONE, CW SPEED, MHz/MEM GRP, MIC GAIN, NB LEVEL, RF POWER, STEP | MHz/ MEM GRP |
| 058 | MIC SCAN | Enables/disables scanning access via the microphone's UP/DWN keys | ON/OFF | ON |
| 059 | MIC SEL | The choice of the equipment which connects to the MIC jack. | NOR/RMT/CAT | NOR |
| 060 | MTR ARX SEL | Select the analog meter display configuration while the transceiver is receiving. | SIG,CTR, VLT, N/A, FS, OFF | SIG |
| 061 | MTR ATX SEL | Selects the analog meter display configuration while the transceiver is transmitting. | PWR, ALC, MOD, SWR, VLT, N/A, OFF | PWR |
| 062 | MTR PEAK HOLD | Enabling/disabling of the "peak hold" function of the ON/OFF meter. | ON/OFF | ON |
| 063 | NB LEVEL | Setting of the blanking level for the IF Noise Blanker. | 0~100 | 50 |
| 064 | OP FILTER1 | Not available at this time. |  |  |
| 065 | PG A | Programming the [A] key (in the Operating Function Row "q" (MFg)) function. | All Multifunction, All Menu Item, MONI, Q.SPL, TCALL, ATC, USER | MONI |
| 066 | PG B | Programming the [B] key (in the Operating Function Row "q" (MFg)) function |  | Q.SPL |
| 067 | PG C | Programming the [C] key (in the Operating Function Row "q" (MFg)) function |  | ATC |
| 068 | PG ACC | Programming the optional MH-59A8J microphone's [ACC] button assignment |  | MONI |
| 069 | PG P1 | Programming the optional MH-59A8J microphone's P1 button assignment |  | Q.SPL |
| 070 | PG P2 | Programming the optional MH-59A8J microphone's P2 button assignment |  | TCALL |
| 071 | PKT 1200 | Adjusts the audio input level from the TNC during 1200bps Packet operation. | 0~100 | 50 |
| 072 | PKT 9600 | Adjusts the audio input level from the TNC during 9600bps Packet operation. | 0~100 | 50 |
| 073 | PKT RATE | Sets the transceiver's circuitry for the Packet baud rate to be used. | 1200/9600(bps) | 1200 bps |
| 074 | PROC LEVEL | Sets the compression level for the AF speech processor in the SSB/AM modes. | 0~100 | 50 |
| 075 | RF POWER SET | Setting of the maximum power level for the current band. | 5~100, 2~100 (UHF) | *2 |
| 076 | RPT SHIFT | Sets the magnitude of the repeater shift. | 0.00~99.99(MHz) | *2 |
| 077 | SCAN MODE | Selects the desired Scan/Resume mode. | TIME/BUSY/STOP | TIME |
| 078 | SCAN RESUME | Sets the delay time for scanning resumption. | 1~10(sec) | 5 sec |
| 079 | SPLIT TONE | Enables/disables split CTCSS/DCS coding | ON/OFF | OFF |
| 080 | SQL/RF GAIN | Selects the configuration of the front panel's SQL/RF knob. | RF-GAIN/SQL | *1 |
| 081 | SSB MIC GAIN | Adjusts the microphone gain level for the SSB mode. | 0~100 | 50 |
| 082 | SSB STEP | Selects the tuning steps for the MEM/VFO CH knob in the SSB mode. | 1 kHz/ 2.5kHz/ 5kHz | 2.5 kHz |
| 083 | TONE FREQ | Setting of the CTCSS Tone Frequency. | 50 Standard CTCSS tones | 88.5 Hz |
| 084 | TOT TIME | Select the Time-Out Timer time. | OFF/1~20(min) | OFF |
| 085 | TUNER/ATAS | Selects the device (FC-30 or ATAS-100/-120) to be controlled via the front panel’s [A](TUNE)key. | OFF/ATAS(HF)/ ATAS(H F&50)/ATAS(ALL)/ TUNER | OFF |
| 086 | TX IF FILTER | Selects the Transmit IF filter. | CFIL/FIL1/FIL2 | CFIL |
| 087 | VOX DELAY | Sets the "hangtime" for the VOX circuitry | 100~3000(ms) | 500 ms |
| 088 | VOX GAIN | Sets the gain of the VOX circuitry's input audio detector. | 1~100 | 50 |
| 089 | XVTR A FREQ | Allows an arbitrary frequency to be set on the display, to allow direct frequency readout during transverter operation | 0,000,00-9999,999,00 (kHz) | - |
| 090 | XVTR B FREQ |  |  | - |
| 091 | XVTR SEL | Enables/disables/selects the antenna port to be used for Transverter operation. | OFF/X VTR A/X VTR B | OFF |

*1 Depends on transceiver version.

*2 Depends on operating band and transceiver version.